# MediSync Project Proposal

Revised for HackForge Logix

| PROJECT POSITIONING | CURRENT PROJECT STATUS |
| --- | --- |
| Primary track: Open Innovation Secondary tracks: AI/ML and Cybersecurity | Wokwi simulation in progress; demonstration sequence requires validation. |

Proposal summary

MediSync is a medication reminder and event-tracking concept for people managing medicines across different schedules and package types. It pairs a planned 12-position home/desk unit with a planned six-compartment portable version. The system is designed to record observable package or position changes around a reminder and leave uncertain events for review. A sensed change is not proof that a dose was swallowed.

Status boundary The current demonstration is simulated in Wokwi. It uses an adjustable HX711 model as a simulated load input and an IR EVENT (SIM) button cue; no physical device, live app or cloud connection is in place. The sequence has not yet been validated, and no ingestion, clinical or adherence outcome has been verified.

Revised 7 October 2026

# Problem Statement

Medication routines can involve several medicines with different schedules and package formats. Bottles, liquid containers and blister packs behave differently, so a single sensing method may not fit every position. A reminder confirms only that an alert was issued; it cannot show whether a person removed a dose or took it.

MediSync explores a physical event record around scheduled reminders. Its goal is to make unresolved reminders easier for a user to review and, in a future connected version, optionally share with an authorized helper. The system must describe sensor observations accurately and must not present them as confirmation of ingestion.

# Existing Solutions

The project compared common approaches against mixed packaging, replacement effort, mechanical complexity and the limits of sensor-based evidence.

| Approach reviewed | Trade-off for this project |
| --- | --- |
| RFID tags on containers | A replacement bottle would need a new tag, adding setup work. |
| Camera for blister packs | Needs space, stable lighting and more processing; not selected for the current prototype. |
| Motorized dispenser or carousel | Adds moving parts and mechanical failure points. |
| Pure pill counting | Does not cover liquid medicines or package changes well. |
| Manual pill-weight entry | Takes effort and may need to be repeated when a package changes. |
| IR sensing for blister packs | Kept as a proposed cue, with foil, material, geometry and alignment limits to validate. |

## Selected market and open-source examples

A few adjacent examples help place the proposal. This is a selective context, not a complete competitor survey or a like-for-like performance comparison.

| Reference | What it illustrates | Relevance and limit |
| --- | --- | --- |
| Hero Help Center / MedMinder | Reminder and caregiver-alert example: Hero Help Center | These are established adjacent patterns. MediSync does not yet have a connected app, and its proposal centers on sensing different package types. |
| Shaztech/Pilldispenser | A public project documents an ESP32 carousel with printable parts, PCB files and firmware. | Useful as an embedded and physical-design reference. Its motorized dispensing approach differs from MediSync's proposed reminder and event-tracking concept. |

MediSync’s design hypothesis is that package-specific load-cell and IR sensing can add an event record across bottles, liquids and blister packs without motorized dispensing or a tag transfer when a similar bottle is replaced. That distinction remains proposed and untested; none of these references validates MediSync’s sensing or proves ingestion.

Design choice: MediSync favors package-aware sensing positions and simple reminders over automated dispensing. This is a concept trade-off, not a claim that the proposed sensing has been proven reliable.

# Proposed Solution

MediSync has two proposed form factors. The renders in the presentation show concepts; neither device is a completed physical build.

| Concept | Capacity and intended use | Planned sensing and controls |
| --- | --- | --- |
| Home / desk unit | 12 positions for a stationary multi-medicine routine: 3 pill/container, 3 liquid and 6 blister positions. | Individual load-cell positions for pill/container and liquid packages; IR sensing at blister positions. Planned ESP32, 0.91-inch OLED, 4 touch switches, 12 position LEDs, buzzer, 1200 mAh / 3.7 V backup battery, USB charging module and antenna. |
| Portable case | 6 compartments for personal use or travel. | Planned ESP32-C3, shared load cell with HX711, display, 3 touch controls, 6 compartment LEDs, buzzer and rechargeable battery with USB-C charging. |

## Reminder and event workflow

| Stage | Intended behavior |
| --- | --- |
| Set up | Register a medicine, assign a position and enter a schedule. |
| Remind | Use the screen, position light and sound. The proposed local timing is a light 5 minutes before the dose, 3 beeps at the scheduled time, then 3 beeps twice at +5 minutes and three times at +10 minutes if unresolved. |
| Sense | Compare the available package or position signal around the reminder. A changed signal is an event candidate, not confirmation of ingestion. |
| Record and review | Record a candidate or leave the reminder unresolved when the expected change is not observed. The user remains responsible for medication decisions. |
| Escalate | A connected app notification to selected helpers is a future option. No app or cloud escalation is connected in the current demo. |

## Medicine registration and replacement

The proposed setup supports QR or barcode entry, manual entry, device-screen controls or a future phone app. After registration, a medicine is assigned to a position. For a similar replacement package, the saved weight profile may be reused after checking it; a materially different package should prompt the user to confirm or recalibrate the profile. The design avoids transferring an RFID tag between bottles.

## Hardware plan and early BOM

The desk concept calls for six load-cell positions, six blister sensing positions and 12 indicators. The portable concept uses one shared load input. These are design targets; sensor selection, calibration, physical integration and the final bill of materials remain to be validated.

| Early notebook estimate | What needs correction before budgeting |
| --- | --- |
| Home / desk: 1,798 noted; listed lines add to 1,807 if a full LED pack is purchased. | The earlier list includes 2 full-bridge cells, 2 half-bridge cells, 4 HX711 amplifiers, 2 IR units and 8+ LEDs. It does not match the current 6 load-cell positions, 6 IR positions and 12 LEDs design target. |
| Portable: 949 subtotal noted. | The amount is from an earlier component list and is not a validated current quote. |

Budget caveat The earlier estimates are incomplete and internally inconsistent; the handwritten lists do not state a currency. Confirm currency, quantities, supplier prices, battery and enclosure requirements, and any omitted modules before using either amount as a project budget.

# Key Features

| Feature | Status in the current demo | Limit |
| --- | --- | --- |
| Scheduled reminders | Simulated with a compressed clock and OLED / buzzer / LED cues. | Wokwi sequence is in progress and must be validated. |
| Weight event candidate | One adjustable Wokwi HX711 model supplies a simulated load input. | Candidate signal only; it is not a physical measurement or proof of ingestion. |
| Blister event cue | Button labeled IR EVENT (SIM) can provide a simulated cue. | No physical IR sensor is connected in the current demo. |
| Event review and unresolved status | Part of the proposed flow; inspect the serial event output after validation. | Do not describe event logging or ingestion as verified until the sequence is observed and captured. |
| App, cloud, adaptive ML, hospital dashboard and TEE | Planned only. | No live app/cloud connection or implemented AI/security layer. |

The current stack is a simulation of selected inputs and reminder cues. The home/desk and portable product concepts are not represented by complete physical devices.

# Technical Approach

The proposed system links a schedule to a reminder, samples the signal available for the assigned position, and records the result as a candidate event or an unresolved reminder. It is a decision-support concept: a user reviews what happened, and the system does not make medication decisions.

| Layer | Current simulation | Planned system |
| --- | --- | --- |
| Schedule and reminder | Example schedule and compressed-clock reminder cues in Wokwi. | User-configured schedules on the device and, later, in an offline app. |
| Inputs | One simulated HX711 load input; optional button cue labeled IR EVENT (SIM). | Package-specific load cells and IR sensing after physical calibration and testing. |
| Event handling | Demonstration sequence intended to produce a candidate for serial inspection; validation pending. | Store event context and unresolved status for user review, with optional sharing only after app and security work. |
| Connectivity | No phone app or cloud connection. | Offline-first phone app is planned; optional cloud AI and care-team dashboard are later possibilities. |

## Current demonstration sequence

The presentation proposes setting the simulated load near 0.50 kg, entering an example schedule and baseline, triggering a reminder with the compressed clock, acknowledging with SELECT, adjusting the input near 0.35 kg, optionally pressing IR EVENT (SIM), then confirming and inspecting the OLED and serial output. This is a validation script, not a verified result. Capture the actual display and serial output before describing the sequence as working.

## Project status and evidence

Wokwi simulation is in progress; its reminder and event sequence still requires validation.

The current load input is the Wokwi HX711 model, and the IR cue is a button; no physical sensor has been validated.

There is no live app/cloud connection. The offline app, adaptive ML, cloud AI, hospital dashboard and TEE are planned, not implemented.

No physical MediSync device has been built, and no clinical, adherence, ingestion or time-saving outcomes have been measured or verified.

# Technology Stack

| Area | Current / proposed technology | Status |
| --- | --- | --- |
| Firmware and simulation | ESP32; Arduino C++; PlatformIO; Wokwi. | Simulation project supplied; validation pending. |
| Display and controls | SSD1306 OLED, NeoPixel ring, buttons and buzzer in the simulated demonstration. | Cues are represented in simulation. |
| Load input | Wokwi HX711 model with one adjustable simulated input. | Not a physical sensor measurement. |
| Blister cue | IR EVENT (SIM) button. | No physical IR sensor connected. |
| Phone app and data | Offline-first app with local data is planned. | Not implemented or connected. |
| AI and care workflow | Adaptive ML, optional cloud AI, hospital dashboard and audit trail are future options. | Planned; requires validation, consent and workflow review. |
| Mobile security | Encryption, role-based access, controlled sharing/deletion and evaluation of TEE on supported phones. | Security roadmap only; not implemented. |

# Expected Impact

If later prototypes prove reliable and usable, MediSync could give people managing multiple medicines and their authorized helpers a clearer view of reminders awaiting follow-up. A portable version could support routines away from home. A future care-team view could surface unresolved events for staff review if it fits approved workflows.

These are expected benefits, not measured results. The team has not measured adherence, clinical outcomes, time saved, usability or sensor false-event rates. Package variation, calibration drift, blister materials, accidental interaction, alert burden, privacy and connectivity all need testing. A sensor event cannot establish that medicine was swallowed, and the patient and care team retain responsibility for decisions.

# Future Scope

| Phase | Next work |
| --- | --- |
| 1. Validate the simulation | Run the Wokwi sequence end to end, confirm reminder and event-state behavior, and save OLED and serial evidence. Update the demo claim to match what is observed. |
| 2. Build and test hardware | Develop desk and portable prototypes; calibrate across real bottle sizes, liquid containers and blister materials. Measure missed and false event rates, reliability and setup effort. |
| 3. Develop the offline app | Create local profiles, schedule setup, event review and user-controlled sharing. Test accessibility, privacy and recovery before adding connectivity. |
| 4. Evaluate adaptive ML | Only after suitable data and consent, test pattern suggestions with user review. Measure errors and keep suggestions separate from medication decisions. |
| 5. Assess connected care and security | Review consent-based cloud AI, care-team workflow, role-based access, audit trails, encryption and TEE feasibility on supported phones before implementation. |
| 6. Rebuild the BOM | Reconcile quantities against the selected design, confirm currency and current supplier prices, and include enclosure, charging, assembly and testing costs. |

MediSync is currently an in-progress simulation supporting a broader Open Innovation concept. The next credible milestone is a validated Wokwi demonstration, followed by measured physical-sensor testing before making reliability or impact claims.